Annexure 1: Bid Specification:

RFB 3268 2026

PROCUREMENT FOR THE SUPPLY, INSTALLATION OF ICT NETWORK SWITCHES, ACCESS POINTS AND CABLING SOLUTION FOR THE LIMPOPO DEPARTMENT OF SOCIAL DEVELOPMENT

TECHNICAL, PRICING AND PREFERENCE POINTS REQUIREMENTS

# Contents

1.	Introduction	4

1.1	Purpose	4

1.2	Background	4

2.	Scope of Bid	4

2.1	Scope of Work	4

2.2	Delivery Address	4

3.	Requirements	4

3.1	Product, Service and Solution Requirements	4

3.2	LAN infrastructure equipment at LDSD	4

3.3	LAN cabling (summary per site)	5

3.3.1	Head Office (Polokwane - Building 1)	5

3.3.2	Head Office (Polokwane - Building 2)	5

3.3.3	Capricorn District	5

3.3.4	Vhembe District (Thohoyandou)	5

3.4	Fibre optic requirements	5

3.4.1	Fibre	5

3.5	Switching requirements	6

3.5.1	Core Switches (Layer 3)	6

3.6	Core Switch Specification	6

3.6.1	Access Switches (Layer 2)	6

3.7	Access Switch Specifications	6

3.8	Wireless Requirements	6

3.8.1	Wireless LAN	6

3.9	Installation Requirements	7

3.9.1	Installation and support	7

3.10	Configuration Requirements	7

3.10.1	Configuration	7

3.11	Testing and Commissioning Requirements	7

3.11.1	Testing and Commissioning	7

3.12	Handover and support documentation Requirements	8

3.12.1	Handover and support documentation	8

4.	Bid Evaluation Stages	8

4.1	Mandatory Administrative responsiveness (Stage 1)	8

4.1.1	Attendance of briefing session	8

4.1.2	Registered Supplier	8

4.2	Technical returnable documents	8

4.2.1	Instruction and evaluation criteria	8

4.2.2	Technical mandatory requirements (Stage 2)	9

4.3	Special Conditions of Contract Verification (Stage 3)	11

4.3.1	Special Conditions of Contract	11

4.3.2	Declaration of compliance and acceptance SCC	16

4.4	Price and Preference Points Evaluation (Stage 4)	16

4.5	Preference Requirements	18

Annex A: Bidder Substantiating Evidence	22

5.	Technical Mandatory Requirement Evidence	22

5.1	Bidder Certification / Affiliation Requirements	22

5.2	Bidder Experience and Capability Requirements	22

5.3	Product / Service Functional Requirements	23

5.4	Special Conditions of Contract Verification	23

5.5	Preference Points Preferential Goals Evidence	23

Annex B: Addendum 1 Service/Product Functional Requirements	25

# Introduction

## Purpose

The purpose of this Request for Bid (RFB) is to invite Suppliers (hereinafter referred to as “bidders”) to submit bids for the supply, install switches, wireless access points (AP) and cabling solution for the Limpopo Department of Social Development (LDSD).

## Background

The supplier is required to install ICT networks infrastructure to Provincial Head Office, Capricorn district and Vhembe district office.

# Scope of Bid

## Scope of Work

The scope of work for the bidders is as follow:

- Installation of 662 Category 6A data points, comprising 326 at the Head Office, 179 at the Capricorn District Office, and 157 at the Vhembe District Office, including the supply and installation of patch leads, fly leads, patch panels, brush panels, blank plates, 25U floor-standing network cabinets, power distribution units (PDUs), trunking, and all associated structured cabling accessories.

- Preparation and submission of LAN and fibre infrastructure layout drawings (Microsoft Visio), including as-built documentation and complete copper and fibre test certification reports.

- Supply, installation and configuration of 31 Layer 2 PoE+ Access Switches, comprising 19 at the Head Office, 8 at the Capricorn District Office, and 4 at the Vhembe District Office; 3 Layer 3 Core Switches (one per site); 124 multimode SFP+ optical transceivers; and 24 × 10G SFP28 optical modules.

- Supply, installation and configuration of 65 Wi-Fi 7 Wireless Access Points, comprising 35 at the Head Office, 15 at the Capricorn District Office, and 15 at the Vhembe District Office, including centralized wireless management, optimisation, and integration with the LAN infrastructure.

- Installation, termination, splicing, testing and commissioning of OM4 multimode with total length of 1500 metres and OS2 single-mode fibre optic infrastructure with a total length 0f 150 metres, including fibre patch panels, patch cords, pigtails, couplers, splice cassettes, conduits, and all associated accessories.

- Configuration of the LAN infrastructure, including VLANs, inter-VLAN routing, IP addressing, DHCP, Quality of Service (QoS), network security, switch management, testing, commissioning, and optimisation.

- Provision of a minimum five (5) year manufacturer warranty on all supplied equipment, together with a workmanship warranty, project handover, and knowledge transfer to the Department's ICT personnel.

## Delivery Address

The address where the required goods / services / works must be delivered is:

Physical Address

Head Office - 76 Schoeman Street, Polokwane Central

Capricorn District Office - 78 Hans Van Rensburg Street, Polokwane Central

Vhembe District Office - Thohoyandou Old Parliament Building

# Requirements

## Product, Service and Solution Requirements

## LAN infrastructure equipment at LDSD

## LAN Cabling (summary per site)

### Head Office (Polokwane - Building 1)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 48 |
| 2nd Floor (Server Room) | 96 |
| 3rd Floor | 62 |

### Head Office (Polokwane - Building 2)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 48 |
| 1st Floor | 24 |
| 2nd Floor | 48 |
| 3rd–5th Floors (per floor) | 24 |

### Capricorn District

| Floor | Network Points |
| --- | --- |
| 3rd Floor | 57 |
| 4th Floor | 65 |
| 5th Floor | 57 |

### Vhembe District (Thohoyandou)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 18 |
| 1st Floor | 25 |
| 2nd Floor | 42 |

General Requirements: CAT6A cabling, patch panels, 25U cabinets, labelling, testing, certification.

## Fibre optic requirements

### Fibre

| Type | Use Case |
| --- | --- |
| OM4 | Internal fibre links |
| OS2 | Inter-building (Head Office only) |
| LC-LC Patch Panels | Connectivity termination |
| OTDR Testing | Fibre certification |
| Power Meter Testing | Signal validation |

## Switching requirements

### Core Switches (Layer 3)

| Site | Quantity |
| --- | --- |
| Head Office | 1 |
| Capricorn District | 1 |
| Vhembe District | 1 |
| Total | 3 |

## Core Switch Specification

| Feature | Minimum Requirement |
| --- | --- |
| Switch Type | Enterprise Managed Layer 3 Core Switch |
| Ports | Minimum 24 × 10G SFP+ ports |
| Switching Capacity | 2 Tbps |
| Layer 2 Features | VLAN, STP/RSTP/MSTP, LACP, LLDP |
| Layer 3 Features | Static Routing, OSPF, VRRP, IPv4/IPv6 |
| QoS | 802.1p, DSCP, Traffic Prioritisation, Rate Limiting |
| Stacking | Physical or Virtual Stacking Supported |
| Management | Web GUI, CLI, SSH, SNMPv3 |
| Power Supply | Internal AC Power Supply |

### Access Switches (Layer 2)

| Sites | Quantity |
| --- | --- |
| Head Office | 19 |
| Capricorn District | 8 |
| Vhembe District | 4 |
| Total | 31 |

## Access Switch Specifications

| Feature | Minimum Requirement |
| --- | --- |
| Switch Type | Enterprise Managed Layer 2 PoE+ Switch |
| Ports | 24 × 1G PoE+ RJ45 |
| Uplink Ports | 4 × 10G SFP+ |
| Switching Capacity | ≥ 176 Gbps. |
| QoS | Voice VLAN, DSCP, Traffic Prioritisation |
| PoE Budget | Minimum 380 W |
| Layer 2 Features | VLAN, STP/RSTP, LACP |
| Layer 3 Features | Static Routing |
| Stacking | Supported |

## Wireless Requirements

### Wireless LAN

| All site | Quantity |
| --- | --- |
| 5 APs per floor | 65 |

| Feature | Specification |
| --- | --- |
| Standard | Wi-Fi 7 Enterprise |
| Bands | 2.4 / 5 / 6 GHz |
| Throughput | ≥ 21 Gbps |
| Security | WPA3 |
| Uplink | 10GbE PoE++ |
| Management | Centralised |

## Installation Requirements

### Installation and support

| Item | Requirement |
| --- | --- |
| Standards | Electrical & Safety Compliance |
| Cabling | Structured & Labelled |
| Fibre | Dedicated conduits |
| Power | Redundant systems |
| Support Period | 60 Months |
| Response Time | 4-hour call, next business day repair |
| Coverage | Nationwide on-site OEM support |

## Configuration Requirements

### Configuration

| Area | Requirement |
| --- | --- |
| VLANs | Data, Voice, Wi-Fi, Management |
| Routing | OSPF, BGP |
| DHCP | Configured services |
| QoS | Traffic prioritisation |
| Security | SSH, SNMPv3, ACLs |
| Monitoring | Syslog, Telemetry |

## Testing and Commissioning Requirements

### Testing and Commissioning

| Phase | Description |
| --- | --- |
| Pre-Commissioning | Initial validation checks |
| Functional Testing | Performance verification |
| Security Testing | Compliance validation |
| Reporting | Formal test reports submission |

## Handover and support documentation Requirements

### Handover and support documentation

| Item | Description |
| --- | --- |
| Audit Forms | Equipment serial numbers |
| Documentation | Certified handover packs |
| Sign-off | Client and provider approval |

# Bid Evaluation Stages

The bid evaluation process consists of Four Stages, according to the nature of the bid. A bidder must qualify for each stage to be eligible to proceed to the next stage of the evaluation. The stages are:

Table 1: Bid Evaluation Stages

| Stage | Description | Applicable for this bid YES/NO |
| --- | --- | --- |
| Stage 1 | Mandatory Administrative Responsiveness | YES |
| Stage 2 | Technical Mandatory Responsiveness | YES |
| Stage 3 | Special Conditions of Contract verification | YES |
| Stage 4 | Price and Preference Points Evaluation | YES |

## Mandatory Administrative responsiveness (Stage 1)

### Attendance of briefing session

- A compulsory on-site briefing session will be held. The bidder must sign the briefing session attendance register using the same information (bidder company name, bidder representative person name and contact details) as submitted in the bidder’s response document. Failure to attend the compulsory briefing session will result in disqualification.

### Registered Supplier

- Only responses from bidders who are registered as a Supplier on National Treasury’s Central Supplier Database (CSD) in terms of National Treasury’s Instruction Note 4A of 2016/17 will be considered for award on this RFB.

## Technical returnable documents

### Instruction and evaluation criteria

- The bidder must comply with ALL the requirements as per the Technical Mandatory Requirements below by providing substantiating evidence in the form of documentation or information, failing which it will be regarded as “NOT COMPLY”.

- The bidder must provide a unique reference number (e.g. binder/folio, chapter, section, page) to locate substantiating evidence in the bid response.

- The bidder must comply with ALL the TECHNICAL MANDATORY REQUIREMENTS for the bid response to proceed to the next stage of the evaluation.

### Technical mandatory requirements (Stage 2)

Table 2: Technical Mandatory Requirements

| Mandatory Requirements | Substantiating evidence of compliance (used to evaluate bid) | Evidence reference (to be completed by bidder) |
| --- | --- | --- |
| 1. Bidder Certification/ Affiliation Requirements |  |  |
| The bidder must be a registered OEM partner/reseller/distributor to provide Cabling, Switches, and Wireless Access Points. | Attach to ANNEX A, copy of a valid documentation (certificate or letter) from OEM as proof that the bidder is a registered partner/reseller/distributor to provide Cabling, Switches, and Wireless Access Points. The certificate/letter must clearly indicate: - The name of the OEM issuing the certificate/letter. - That the bidder is a registered partner/reseller/distributor to provide cabling, switches and network infrastructure. - Date of the certificate/Letter on which it was issued. NOTE (1): SITA reserve the right to verify information provided. | <provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.1> |
| 2. Bidder Experience and Capability Requirements |  |  |
| The bidder must have supplied, installed, configured switches, wireless Access Points and cable solution to at least Two (2) customers within the past Five (5) years from the publication date of this bid. | Attach to ANNEX A, reference details and/or reference letter/s from at least Two (2) customers to whom the supply, installation, configuration of switches, wireless Access Points and cable solution was delivered in past Five (5) years from the publication date of this bid. NOTE (1): The Bidder must provide all of the following information when completing Table 6: - Company name; and - Contact person, telephone and/or e-mail address; and - Project scope of Work; and - Project start and End date. NOTE (2): The reference letter/s should be on the referees’ company letterhead and include all of the following information: - Company Name; and - Contact Person, telephone and/or e-mail address; and - Project Scope of Work; and - Project Start and End date. NOTE (3): SITA reserves the right to verify information provided. NOTE (4): Failure to submit reference letter/s and/or to complete Table 6 fully as indicated above will result in disqualification. | <provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.2, table 6> |
| 3. Product, Service and Functional Requirements |  |  |
| The bidder must confirm compliance to the Product, Service and Solution requirements. | The bidder must confirm that they comply with the Product, Service Functional Requirements by completing, signing and submitting Annex B: Addendum 1. NOTE: SITA reserves the right to verify information provided. | <Provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.3 and Annex B> |
| 4. Special Conditions of Contract Verification |  |  |
| Bidder must accept ALL the Special Conditions of contract. | The Bidder must accept ALL the Special Conditions of Contract by completing and signing the declaration of Acceptance in the Declaration of Compliance and Acceptance under the Special Conditions (Section 4.3.2). NOTE (1): Failure to accept ALL the Special Conditions of Contract will result in disqualification. | <provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.4> |

## Special Conditions of Contract Verification (Stage 3)

- The successful supplier will be bound by Government Procurement: General Conditions of Contract (GCC) as well as this Special Conditions of Contract (SCC), which will form part of the signed contract with the successful Supplier. However, SITA reserves the right to include or waive the condition in the signed contract.

- SITA reserves the right to:

  - Negotiate the conditions; or

  - Automatically disqualify a bidder for not accepting these conditions; or

- If the bidder qualifies the proposal with own conditions and does not specifically withdraw such own conditions when called upon to do so, SITA will invoke the rights reserved in accordance with subsection 4.3. (b) above.

### Special Conditions of Contract

#### Contracting Conditions

- Formal Contract - The supplier must enter a formal written contract (agreement) with SITA.

- Right to Audit - SITA reserves the right, before entering a contract, to conduct or commission an external service provider to conduct a financial audit or probity to ascertain whether a qualifying bidder has the financial wherewithal or technical capability to provide the goods and services as required by this tender.

#### Delivery Address

- The supplier must deliver the required products or services at as indicated in Section 2.2, Delivery Address

#### Delivery schedule

- The scope of work (Section 2.1) must be completed within 4 weeks after the award.

- The Supplier is responsible to perform the work as outlined in the following Breakdown Structure (WBS):

| WBS | Statement of Work | Delivery Timeframe |
| --- | --- | --- |
|  | The bidder must supply, install, and configure network switches and access points (APs), as well as supply and install structured LAN cabling. | 4 weeks after the award. |

#### Supplier Performance Reporting

- Reports need to be provided on a weekly basis to SITA/Client during the design, installation and implementation phase of the project; weekly written reports are to be presented to the SITA/Client on the progress of the preceding week until installation process has been completed.

#### Certification, Expertise and Qualification

- The bidder certifies that:

  - it has the necessary expertise, skill, qualifications and ability to undertake the work required or defined in the scope and BOM.

  - it is committed to provide the Products or Services; and

  - perform all obligations detailed herein without any interruption to the Customer

  - it has been certified for the Products and Services required

- The bidder must ensure that work or service is performed by a person who is certified by Original Equipment Manufacturer.

#### Logistical Conditions

- Hours of Work

  - Office hours are defined as business working hours of the customer and is Mondays to Fridays between 07:30 and 16:00

  - After hours of the customer during weekdays are from 16:00 to 07:30

- Client environment

  - If SITA grants the bidder access to SITA's Environment including hardware, software, internet facilities, data, telecommunication facilities and/or network facilities remotely, the Supplier must adhere to SITA's relevant policies and procedures (which policy and procedures are available to the Supplier on request) or in the absence of such policy and procedures, in terms of, best industry practice

- Tools of Trade

  - The bidder is expected to use its own resources (cell phone, laptops etc) to communicate with its own offices or outside of the SITA/Client buildings, including all tools and equipment to render the services effectively.

#### Regulatory, Quality and Standards

- Products used to deliver the goods /services must comply with ISO/IEC General Quality Standards, ISO27001.

- Bidders must for the duration of the contract ensure compliance with General Quality Standards, ISO 9001 and Protection of Personal Information Act (POPIA).

- SITA Regulations 12.3 require that before a department concludes a contract, the Agency must conduct s standard (MIOS) certification in respect of the goods or services in question. Bidders are required to attach SITA PRODUCT CERTIFICATES for all offered products. These certificates can be obtained from the OEM. Refer 	to the SITA Product Certification website for more information www.sita.co.za/prodcert.htm.

#### Personnel Security Clearance

  - Company security screening: The supplier may be required to undergo a company security screening conducted by the State Security Agency (SSA). Should the SSA find the supplier not suitable after the conduct of the security screening, the business relationship will be terminated. The following documentation will be required for the company security screening process to be conducted:

    - Copy of company registration documentation.

    - Copy(ies) of identity documentation of Director(s), Member(s) or Trustee(s);

    - Copy of valid tax clearance certificate.

  - Security suitability check for individuals: SITA may, at its own discretion and in line with its policies and procedures, require employees of the supplier to be subjected to a security suitability check before commencement of a project or delivering of a service. The security suitability check is conducted by SITA in order to ensure that individuals meet the minimum-security requirements and also to verify personal information. The supplier will be required to replace any employee(s) who is found to be not suitable after the conduct of the security screening. The following documentation will be required for the security suitability check:

    - Copy of identity document.

    - Copy(ies) of qualification(s) if SITA requires verification thereof.

    - Fingerprints – will be taken electronically.

    - Signed consent form for the conduct of background checks.

  - Security clearance: A security clearance, issued by either the SSA or Defence Intelligence (DI) is required if any employee of the supplier will have or may gain access to classified information throughout the duration of the project or in the process of delivering a service. The level of security clearance required – Confidential, Secret or Top Secret, will be determined at the sole discretion of SITA. The supplier will have to replace any employee who do not qualify for a security clearance or is found not suitable by the SSA or DI. The following documentation will be required for the security clearance process:

    - Completed Z204 or DD1057 security clearance application form.

    - Fingerprints.

    - Personal documentation of the applicant, including but not limited to, identity document, passport, marriage certificate (if applicable), divorce order (if applicable), qualifications, salary advice and bank statements.

#### Confidentiality and non -disclosure conditions

- The Supplier, including its management and staff, must before commencement of the Contract, sign a non-disclosure agreement regarding Confidential Information

- Confidential Information means any information or data, irrespective of the form or medium in which it may be stored, which is not in the public domain, and which becomes available or accessible to a Party as a consequence of this Contract, including information or data which is prohibited from disclosure by virtue of:

  - the Promotion of Access to Information Act, 2000 (Act no. 2 of 2000);

  - being clearly marked "Confidential" and which is provided by one Party to another Party in terms of this Contract.

  - being information or data, which one Party provides to another Party or to which a Party has access because of Services provided in terms of this Contract and in which a Party would have a reasonable expectation of confidentiality.

  - being information provided by one Party to another Party in the course of contractual or other negotiations, which could reasonably be expected to prejudice the right of the non-disclosing Party.

  - being information, the disclosure of which could reasonably be expected to endanger a life or physical security of a person.

  - being technical, scientific, commercial, financial and market-related information, know-how and trade secrets of a Party.

  - being financial, commercial, scientific or technical information, other than trade secrets, of a Party, the disclosure of which would be likely to cause harm to the commercial or financial interests of a non-disclosing Party; and

  - being information supplied by a Party in confidence, the disclosure of which could reasonably be expected either to put the Party at a disadvantage in contractual or other negotiations or to prejudice the Party in commercial competition; or

  - information the disclosure of which would be likely to prejudice or impair the safety and security of a building, structure or system, including, but not limited to, a computer or communication system; a means of transport; or any other property; or a person; methods, systems, plans or procedures for the protection of an individual in accordance with a witness protection scheme; the safety of the public or any part of the public; or the security of property; information the disclosure of which could reasonably be expected to cause prejudice to the defence of the Republic; security of the Republic; or international relations of the Republic; or plans, designs, drawings, functional and technical requirements and specifications of a Party, but must not include information which has been made automatically available, in terms of the Promotion of Access to Information Act, 2000; and information which a Party has a statutory or common law duty to disclose or in respect of which there is no reasonable expectation of privacy or confidentiality;

- Notwithstanding the provisions of this Contract, no Party is entitled to disclose Confidential Information, except where required to do so in terms of a law, without the prior written consent of any other Party having an interest in the disclosure.

- Where a Party discloses Confidential Information which materially damages or could materially damage another Party, the disclosing Party must submit all facts related to the disclosure in writing to the other Party, who must submit information related to such actual or potential material damage to be resolved as a dispute.

- Parties may not, except to the extent that a Party is legally required to make a public statement, make any public statement or issue a press release which could affect another Party, without first submitting a written copy of the proposed public statement or press release to the other Party and obtaining the other Party's prior written approval for such public statement or press release, which consent must not unreasonably be withheld.

#### Guarantee and warranties

- The supplier confirms that:

  - The warranty of goods supplied under this contract remains valid for the duration of the contract after the goods were delivered, installed and commissioned with a sign off, including the clients signature

  - as at Commencement Date, it has the rights, title and interest in and to the Product or Services to deliver such Product or Services in terms of the Contract and that such rights are free from any encumbrances whatsoever.

  - the Product is in good working order, free from Defects in material and workmanship, and substantially conforms to the Specifications, for the duration of the Warranty period.

#### Intellectual Property Rights

- SITA retains all Intellectual Property Rights in and to SITA's Intellectual Property. As of the Effective Date, the Supplier is granted a non-exclusive license, for the continued duration of this Contract, to perform any lawful act including the right to use, copy, maintain, modify, enhance and create derivative works of SITA's Intellectual Property for the sole purpose of providing the Products or Services to SITA pursuant to this Contract; provided that the Supplier must not be permitted to use SITA's Intellectual Property for the benefit of any entities other than SITA without the written consent of SITA, which consent may be withheld in SITA's sole and absolute discretion. Except as otherwise requested or approved by SITA, which approval is in SITA's sole and absolute discretion, the Supplier must cease all use of SITA's Intellectual Property, at of the earliest of:

  - termination or expiration date of this Contract.

  - the date of completion of the Services; and

  - the date of rendering of the last of the Deliverables

- If so, required by SITA, the Supplier must certify in writing to SITA that it has either returned all SITA Intellectual Property to SITA or destroyed or deleted all other SITA Intellectual Property in its possession or under its control

- SITA, at all times, owns all Intellectual Property Rights in and to all Bespoke Intellectual Property.

- Save for the license granted in terms of this Contract, the Supplier retains all Intellectual Property Rights in and to the Supplier’s pre-existing Intellectual Property that is used or supplied in connection with the Products or Services

#### General

- The supplier will be bound by Government Procurement: General Conditions of Contract.

- (GCC) as well as this Special Conditions of Contract (SCC), which will form part of the signed contract with the Supplier. However, SITA reserves the right to include or waive the condition in the signed contract.

- SITA reserves the right to:

  - Negotiate the conditions, or

  - Automatically disqualify a bidder for not accepting these conditions, or

  - Before entering a contract, conduct or commission an external service provider to audit or conduct probity to ascertain whether a qualifying bidder has the technical capability to provide the goods and services as required by this tender.

#### Counter Conditions

- Bidders’ attention is drawn to the fact that amendments to any of the Bid Conditions or setting of counter conditions by bidders may result in the invalidation of such bids.

#### Fronting

- The SITA supports the spirit of Broad Based Black Economic Empowerment and recognizes that real empowerment can only be achieved through individuals and businesses conducting themselves in accordance with the Constitution and in an honest, fair, equitable, transparent and legally compliant manner. Against this background the SITA will not condone any form of fronting.

- The SITA, in ensuring that bidders conduct themselves in an honest manner will, as part of the bid evaluation processes, conduct or initiate the necessary enquiries/investigations to determine the accuracy of the representation made in bid documents. Should any of the fronting indicators as contained in the Guidelines on Complex Structures and Transactions and Fronting, issued by the Department of Trade and Industry, be established during such enquiry/investigation, the onus will be on the bidder / contractor to prove that fronting does not exist. Failure to do so within a period of 14 days from date of notification may invalidate the bid / contract and may also result in the restriction of the bidder/contractor to conduct business with the public sector for a period not exceeding ten (10) years, in addition to any other remedies SITA may have against the bidder/contractor concerned.

#### Supplier Due Diligence

- SITA reserves the right to conduct supplier due diligence prior to final award or at any time during the Contract period and this may include pre-announced / non-announced site visits. During the due diligence process the information submitted by the bidder will be verified and any misrepresentation thereof may disqualify the bid or Contract in whole or parts thereof.

#### Preference Goal Requirements conditions

- The Bidder’s commitment for the Preference Goal Requirements in this tender will be legally binding, and the Bidder needs to perform against their commitment for the duration of the contract which will form part of the Contractual Agreement.

- The Bidder must sustain or improve the company’s BBBEE Level for the duration of the contact which will form part of the Contractual Agreement.

- Performance of Preference Goal Requirements will be determined annually. Bidders must submit their Preference status report indicating progress against the Bidder’s Preferential commitments within 30 days of the yearly anniversary of the contract.

- Bidders need to keep auditable substantive records / evidence and upon request by SITA/Department must be made available for audit and, or due diligence purposes.

- SITA reserves the right to require from a Bidder, either before a bid is adjudicated or at any time subsequently, to substantiate any claim with regards to preferences, in any manner required by SITA.

- SITA reserves the right to verify information / evidence provided by the Bidder.

- SITA/Department reserves the right to introduce a penalty of 1% of the overall annual year spent by SITA/Department for the prior year if the Bidder fails to comply to paragraphs (a), (b) and (c) above.

### Declaration of compliance and acceptance SCC

I (we), the bidder hereby declare that I (we) accept ALL the Special Conditions of Contract as specified in par 4.3.1 above and shall comply with all stated obligations:

Name of Bidder:_____________________________	Signature: _________________________

Date:______________

## Price and Preference Points Evaluation (Stage 4)

    - Costing and Preference Evaluation

- In terms of the SITA Preferential Procurement Policy (PPP), the following preference point system is applicable to all Bids:

  - the 80/20 system (80 Price, 20 Specific Goals) for requirements with a Rand value of up to R50 000 000 (all applicable taxes included); or

- The Applicable Preference Point system for this tender is the 80/20 preference point system.

- Points for this tender shall be awarded for:

  - Price; and

  - Preference points for specific goals.

- The maximum points for this tender will be allocated as follows, subject to par.2.

Table 3: Points Allocation

| Description | Points |
| --- | --- |
| Price | 80 |
| Preference points for specific goals | 20 |
| Total points for Price and preference points for specific goals | 100 |

    - Costing and Pricing Conditions

            - South African Pricing - The total price must be VAT inclusive and be quoted in South African Rand (ZAR).

            - Total Price

  - All quoted prices are the total price for the entire scope of required services and deliverables to be provided by the bidder.

  - All additional costs as well as cost of delivery, labour, S&T, overtime, etc. must be included in this bid.

  - All services, accessories, upgrades and options required by the solution or specified by the client must be included in the quoted price. If not included, suppliers will be required to supply these accessories at no cost to the client.

  - SITA reserves the right to negotiate pricing with the successful bidder prior to the award as well as envisaged quantities

            - These conditions will form part of the Contract between SITA and the bidder. However, SITA reserves the right to include or waive the condition in the Contract.

            - The bidder must complete the declaration of acceptance as per par 4.4.5 below by marking with an “X” either “ACCEPT ALL”, or “DO NOT ACCEPT ALL”, failing which the declaration will be regarded as “DO NOT ACCEPT ALL” and the bid will be disqualified.

            - Rate of Exchange Pricing Information

Provide the TOTAL BID PRICE for the duration of Contract and clearly indicate the Local Price and Foreign Price, where –

- Local Price means the portion of the TOTAL price that is NOT dependent on the Foreign Rate of Exchange (ROE) and.

- Foreign Price means the portion of the TOTAL price that is dependent on the Foreign Rate of Exchange (ROE).

- Exchange Rate means the ROE (ZA Rand vs foreign currency) as determined at time of bid.

    - Bid Exchange Rate Conditions

The bidders must use the exchange rate provided below to enable SITA to compare the prices provided by using the same exchange rate:

| Foreign currency | South African Rand (ZAR) exchange rate |
| --- | --- |
| 1 US Dollar | R16.48 |
| 1 Euro | R18.80 |
| 1 Pound | R22.05 |

Note (1):

This bid is subject to ROE.

    - Bid Pricing Schedule

Bidders must complete the bid pricing schedule in the Excel spreadsheet format provided and upload this as part of their submission.

    - Declaration of Acceptance

|  | ACCEPT ALL | DO NOT ACCEPT ALL |
| --- | --- | --- |
| - The bidder declares to ACCEPT ALL the Costing and Pricing conditions as specified in par 4.4.2 above by indicating with an “X” in the “ACCEPT ALL” column, or - The bidder declares to NOT ACCEPT ALL the Costing and Pricing Conditions as specified in par 4.4.2 above by - - Indicating with an “X” in the “DO NOT ACCEPT ALL” column, and. - Provide reason and proposal for each of the condition not accepted. |  |  |
| Comments by bidder: Provide the condition reference, the reasons for not accepting the condition. |  |  |

## Preference Requirements

- The bidder must complete in full all the PREFERENCE requirements.

- Allocation of points per requirements: The points allocation of bidders’ responses to the requirements will be determined by the completeness, relevance and accuracy of substantiating evidence.

- Points will be allocated for each PREFERENCE requirement as per the criteria set in each section in the table 5 below.

- The bidder must provide a unique reference number (e.g. binder/folio, chapter, section, page) to locate substantiating evidence in the bid response. During evaluation, SITA reserves the right to treat substantiation evidence that cannot be in the bid response, as “NOT COMPLY”. The evidence needs to be attached to ANNEX A.

- Preference Goal Requirements

  - The applicable Preference Point system for this tender and points claimed is 80/20.

  - The specific Preferential Goal Requirements for this tender is indicated in table 4 below.

  - The Bidder must complete 80/20 preference point system and submit proof or documentation required in terms of this tender.

  - The Bidder must indicate their commitment to claim points for each of the preference points by signing at par 4.5 in the Invitation to Bid document.

  - Failure on the part of a bidder to submit proof or documentation required or to comply to paragraph (d) above in terms of this tender to claim preference points for the Preference Goal Requirements for this tender, will be interpreted to mean that preference points are not claimed.

  - The Bidder’s commitment for the Preference Goal Requirements in this tender will be legally binding, and the Bidder needs to perform against their commitment for the duration of the contract which will form part of the Contractual Agreement.

  - The Bidder must sustain or improve the company’s B-BBEE Level for the duration of the contact which will form part of the Contractual Agreement.

  - Performance of Preference Goal Requirements will be determined annually. Bidders must submit their Preference status report to SITA indicating progress against the Bidder’s Preferential commitments within 30 days after each quarter from the commencement date of the contract.

  - Bidders need to keep auditable substantive records / evidence and upon request by SITA must be made available for audit and, or due diligence purposes.

  - SITA reserves the right to require from a Bidder, either before a bid is adjudicated or at any time subsequently, to substantiate any claim with regards to preferences, in any manner required by SITA.

  - SITA reserves the right to verify information / evidence provided by the Bidder.

  - SITA reserves the right to introduce a penalty of 1% of the overall annual year spent by SITA for the prior year if the Bidder fails to comply to paragraphs (f), (g) and (h) above.

Table 4: Preference Goal Requirements (Specific Goals)

| Preference Goal Requirement # | Preferential Goal Requirements | Preferential Goal Requirements |  |
| --- | --- | --- | --- |
|  | Preferential Goal Requirements allocated for this tender | Substantiating evidence and evidence reference to be completed by bidder. Evaluation per requirement: Each requirement indicated in the table below must be completed and points will be allocated based on the evidence required below | Evidence Reference |
|  | B-BBEE Requirements |  |  |
| 1) | B-BBEE Requirements Promotion of Transformational Objectives. | Evidence: The Bidder must provide a copy of the following relevant evidence for the Preferential Goal points which the Bidder qualifies for: - Columns A, B, C and D in table 5 # Copy of relevant proof of the following to confirm the B-BBEE status of the contributor as defined in the Broad-Based Black Economic Empowerment Act: # B-BBEE certificate (from a SANAS Accredited Agency); # or # Sworn affidavit in the format provided by CIPC - Applicable to EMEs and QSEs only; and/ or - Column D in table 5 # Copy of South African Identification Document (ID); and/ or - Column E in table 5 # Copy of Medical Certificate clearly indicating the disability in line with the B-BBEE status claimed as defined in the Broad-Based Black Economic Empowerment Act. Points allocation: Points will be allocated for bidders that meets the requirements as indicated in table 5. | <provide unique reference to locate substantiating evidence in the bid response – Annex A, par 5.3> |

Table 5: B-BBEE Points as part of the Preference Goal requirements (Preferential Goal Requirements for (80/20) system)

Note: Bidder to select the section for points they wish to claim (Mark as Y=Yes) in the table below.

|  |  |  |  | Ownership |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference # | Contributor Level as defined in the Broad-Based Black Economic Empowerment Act | EME/QSEs | Black Owned (BO) (51% or more) | Black Woman Owned (BWO) (More than 30%) | Youth Owned | Owned by People living with disabilities | Score | Bidder to select the section for points they wish to claim (Mark as Y= Yes) |  |
|  |  |  | (A) | (B) | (C) | (D) | (E) | (F) |  |  |
|  | 1 | Level 1 | 6 | 4 | 4 | 4 | 2 | 20 |  |  |
|  | 2 | Level 1 | 6 | 4 | 2 | 2 | 0 | 14 |  |  |
|  | 3 | Level 1 | 6 | 4 | 2 | 0 | 0 | 12 |  |  |
|  | 4 | Level 1 | 6 | 4 | 0 | 0 | 0 | 10 |  |  |
|  | 5 | Level 2 and 3 | 4 | 2 | 1 | 1 | 1 | 9 |  |  |
|  | 6 | Level 2 and 3 | 4 | 2 | 1 | 1 | 0 | 8 |  |  |
|  | 7 | Level 2 and 3 | 4 | 2 | 1 | 0 | 0 | 7 |  |  |
|  | 8 | Level 2 and 3 | 4 | 2 | 0 | 0 | 0 | 6 |  |  |
|  | 9 | Level 4 and 5 | 2 | 1 | 0,5 | 0,5 | 0,5 | 4,5 |  |  |
|  | 10 | Level 4 and 5 | 2 | 0,5 | 0,5 | 0,5 | 0 | 3,5 |  |  |
|  | 11 | Level 4 and 5 | 2 | 0,5 | 0,5 | 0 | 0 | 3 |  |  |
|  | 12 | Level 4 and 5 | 2 | 0,5 | 0 | 0 | 0 | 2,5 |  |  |
|  | 13 | Level 6 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 14 | Level 7 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 15 | Level 8 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 16 | Non-Contributor | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | Total Maximum Score Allocation: |  | 20 |  |  |  |  |  |  |  |

Bidder Substantiating Evidence

# Technical Mandatory Requirement Evidence

## Bidder Certification / Affiliation Requirements

- Attach copy of a valid documentation (certificate or letter) from OEM as proof that the bidder is a registered partner/reseller/distributor to provide Cabling, Switches, and Wireless Access Points. (Refer par 4.2.2 above).

The certificate/letter must clearly indicate:

- The name of the OEM issuing the certificate/letter.

- That the bidder is a registered partner/reseller/distributor to provide cabling, switches and network infrastructure.

- Date of the certificate/Letter on which it was issued.

NOTE (1):

SITA reserve the right to verify information provided.

## Bidder Experience and Capability Requirements

- Complete table below, noting that:

  - Attach reference details and/or reference letter/s from at least two (2) customers to whom the supply, installation, configuration of switches, wireless Access Points and cable solution was delivered in past five (5) years from the publication date of this bid.

  - Project end-date must be current or not older than five (5) years from date this bid is advertised.

  - Scope of work must be related.

Table 6: References

| No | Company Name | Reference person name, contact details | Project Scope of Work | Project start and end date |
| --- | --- | --- | --- | --- |
| 1 | <Company name> | <Person Name> <Tel> <email> | < Provide scope details of a project from a customer to whom the supply, installation, configuration of switches, wireless access point and cable solution was delivered in the past five years from the publication of this bid>. | Start Date: End Date: |
| 2 | <Company name> | <Person Name> <Tel> <email> | < Provide scope details of a project from a customer to whom the supply, installation, configuration of switches, wireless access point and cable solution was delivered in the past five years from the publication of this bid>. | Start Date: End Date: |

NOTE (1):

The Bidder must provide all of the following information when completing Table 6:

- Company Name; and

- Contact Person, telephone and/or e-mail address; and

- Project Scope of Work; and

- Project Start and End date.

NOTE (2):

The reference letter/s should be on the referees’ company letterhead and include all of the following 	information:

- Company Name; and

- Contact person, telephone and/or e-mail address; and

- Project scope of Work; and

- Project start and End date.

NOTE (3):

SITA reserves the right to verify information provided.

NOTE (4):

Failure to submit reference letter/s and/or to complete Table 6 fully as indicated above will result in 	disqualification.

## Product / Service Functional Requirements

The bidder must confirm that they comply with the Product / Service Functional Requirements by 	completing Annex B: Addendum 1.

## Special Conditions of Contract Verification

The Bidder must accept ALL the Special Conditions of Contract by completing and signing the declaration of Acceptance in the Declaration of Compliance and Acceptance under the Special Conditions (Section 4.3.2).

NOTE (1):

Failure to accept ALL the Special Conditions of Contract will result in disqualification.

## Preference Points Preferential Goals Evidence

Evidence:
The Bidder must provide a copy of the following relevant evidence for the Preferential Goal points which the Bidder qualifies for:

- Columns A, B, C and D in Table 5

# Copy of relevant proof of the following to confirm the B-BBEE status of the contributor as defined in the Broad-Based Black Economic Empowerment Act:

# B-BBEE certificate (from a SANAS Accredited Agency);

# or

# Sworn affidavit in the format provided by CIPC - Applicable to EMEs and QSEs only;

# and/ or

- Column D in Table 5

# Copy of South African Identification Document (ID);

# and/ or

- Column E in Table 5

# Copy of Medical Certificate clearly indicating the disability in line with the B-BBEE status claimed as defined in the Broad-Based Black Economic Empowerment Act.

Points allocation:
Points will be allocated for bidders that meets the requirements as indicated in Table 5 in section 4.5.

Addendum 1 Service/Product Functional Requirements

NB: The bidder must confirm that they comply with the following Service / Product Functional Requirements as indicated below as this will be legal contractual binding:

- Lan Cabling (summary per site)

- Head Office (Polokwane - Building 1)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 48 |
| 2nd Floor (Server Room) | 96 |
| 3rd Floor | 62 |

- Head Office (Polokwane - Building 2)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 48 |
| 1st Floor | 24 |
| 2nd Floor | 48 |
| 3rd–5th Floors (per floor) | 24 |

- Capricorn District

| Floor | Network Points |
| --- | --- |
| 3rd Floor | 57 |
| 4th Floor | 65 |
| 5th Floor | 57 |

- Vhembe District (Thohoyandou)

| Floor | Network Points |
| --- | --- |
| Ground Floor | 18 |
| 1st Floor | 25 |
| 2nd Floor | 42 |

General Requirements: CAT6A cabling, patch panels, 22U/27U cabinets, labelling, testing, certification.

- Fibre optic requirements

- Fibre

| Type | Use Case |
| --- | --- |
| OM4 | Internal fibre links |
| OS2 | Inter-building (Head Office only) |
| LC-LC Patch Panels | Connectivity termination |
| OTDR Testing | Fibre certification |
| Power Meter Testing | Signal validation |

- Switching requirements

- Core Switches (Layer 3)

| Site | Quantity |
| --- | --- |
| Head Office | 1 |
| Capricorn District | 1 |
| Vhembe District | 1 |
| Total | 3 |

- Core Switch Specification

| Feature | Minimum Requirement |
| --- | --- |
| Switch Type | Enterprise Managed Layer 3 Core Switch |
| Ports | Minimum 24 × 10G SFP+ ports |
| Switching Capacity | ≥ 480 Gbps |
| Layer 2 Features | VLAN, STP/RSTP/MSTP, LACP, LLDP |
| Layer 3 Features | Static Routing, OSPF, VRRP, IPv4/IPv6 |
| Switching Capacity | ≥ 480 Gbps |
| QoS | 802.1p, DSCP, Traffic Prioritisation, Rate Limiting |
| Stacking | Physical or Virtual Stacking Supported |
| Management | Web GUI, CLI, SSH, SNMPv3 |
| Power Supply | Internal AC Power Supply |
| Mounting | 19-inch Rack Mount |
| Warranty | Minimum 5 Years |

Access Switches (Layer 2)

| Sites | Quantity |
| --- | --- |
| Head Office | 19 |
| Capricorn District | 8 |
| Vhembe District | 4 |
| Total | 31 |

- Access Switch Specifications

| Feature | Minimum Requirement |
| --- | --- |
| Switch Type | Enterprise Managed Layer 2 PoE+ Switch |
| Ports | 24 × 1G PoE+ RJ45 |
| Uplink Ports | 4 × 10G SFP+ |
| Switching Capacity | ≥ 128 Gbps. |
| QoS | Voice VLAN, DSCP, Traffic Prioritisation |
| PoE Budget | Minimum 380 W |
| Layer 2 Features | VLAN, STP/RSTP, LACP |
| Layer 3 Features | Static Routing |
| Stacking | Supported |
| Warranty | Minimum 5 Years |

- Wireless Requirements

Wireless LAN

| All site | Quantity |
| --- | --- |
| 5 APs per floor | 65 |

| Feature | Specification |
| --- | --- |
| Standard | Wi-Fi 7 Enterprise |
| Bands | 2.4 / 5 / 6 GHz |
| Throughput | ≥ 21 Gbps |
| Security | WPA3 |
| Uplink | 10GbE PoE++ |
| Management | Centralised |

- Installation Requirements

Installation and Support

| Item | Requirement |
| --- | --- |
| Standards | Electrical & Safety Compliance |
| Cabling | Structured & Labelled |
| Fibre | Dedicated conduits |
| Power | Redundant systems |
| Support Period | 60 Months |
| Response Time | 4-hour call, next business day repair |
| Coverage | Nationwide on-site OEM support |

- Configuration Requirements

Configuration

| Area | Requirement |
| --- | --- |
| VLANs | Data, Voice, Wi-Fi, Management |
| Routing | OSPF, BGP |
| DHCP | Configured services |
| QoS | Traffic prioritisation |
| Security | SSH, SNMPv3, ACLs |
| Monitoring | Syslog, Telemetry |

- Testing and Commissioning Requirements

Testing and Commissioning

| Phase | Description |
| --- | --- |
| Pre-Commissioning | Initial validation checks |
| Functional Testing | Performance verification |
| Security Testing | Compliance validation |
| Reporting | Formal test reports submission |

- Handover and support documentation Requirements

Handover and support documentation

| Item | Description |
| --- | --- |
| Audit Forms | Equipment serial numbers |
| Documentation | Certified handover packs |
| Sign-off | Client and provider approval |

I, the bidder (Full names) …………………………………………………. representing (company name) …………………………………………………………….. Hereby confirm that I comply with the above Service/Product Functional Requirements and understand that it will form part of the contract and is legally binding.

Thus done and signed at ……………………………………. On this………day of……………….20….

……………………………….

Signature

Designation: